K.POOJA SREE

192512245

ECA1403 EMBEDDED SYSTEMS

EXPERIMENT 3

# I2C-BASED HEALTH MONITORING NODE FOR PULSE AND TEMPERATURE ACQUISITION

# ➔ PROGRAM:

#include <Wire.h>

#define LM75_ADDRESS 0x48

#define PULSE_PIN 8

unsigned long pulseCount = 0;

unsigned long lastTempTime = 0;

unsigned long lastBpmTime = 0;

int previousPulseState = LOW;

float readTemperature()

{

Wire.beginTransmission(LM75_ADDRESS);

Wire.write(0x00);

Wire.endTransmission();

Wire.requestFrom(LM75_ADDRESS, 2);

if (Wire.available() >= 2)

{

byte msb = Wire.read();

byte lsb = Wire.read();

int16_t rawTemperature =

((int16_t)msb << 8) | lsb;

return rawTemperature / 256.0;

}

return -999.0;

}

void setup()

{

Serial.begin(9600);

Wire.begin();

pinMode(PULSE_PIN, INPUT);

Serial.println("I2C HEALTH MONITORING NODE");

Serial.println("---------------------------");

}

void loop()

{

unsigned long currentTime = millis();

/* Pulse counting */

int currentPulseState = digitalRead(PULSE_PIN);

if (currentPulseState == HIGH &&

previousPulseState == LOW)

{

pulseCount++;

}

previousPulseState = currentPulseState;

/* Temperature acquisition every 1 second */

if (currentTime - lastTempTime >= 1000)

{

lastTempTime = currentTime;

float temperature = readTemperature();

Serial.print("Temperature: ");

if (temperature > -100)

{

Serial.print(temperature, 1);

Serial.println(" C");

}

else

{

Serial.println("Sensor Error");

}

}

/* BPM calculation every 5 seconds */

if (currentTime - lastBpmTime >= 5000)

{

lastBpmTime = currentTime;

float bpm = (pulseCount * 60.0) / 5.0;

Serial.print("Pulse Rate: ");

Serial.print(bpm, 0);

Serial.println(" BPM");

Serial.println("---------------------------");

pulseCount = 0;

}

}

OUTPUT:

REFLECTIONS:

Through this experiment, I learned how to design a health monitoring circuit using Arduino Uno in Proteus. I understood the importance of pull-up resistors and power terminals in circuit connections. This experiment improved my knowledge of circuit simulation, I²C communication, and troubleshooting techniques.